7. UV Module – Technical Details

7.1 Wavelength & Power

The ES-AQUA UV module operates at 254 nm UV-C, the globally recognized standard wavelength for water disinfection.

Typical performance parameters:

- UV-C LED or tube: 1–5 W optical output

- Electrical power: 5–12 W (depending on LED type)

- Target UV dose: ≥ 40 mJ/cm²

This dose is sufficient to inactivate:

- bacteria (e.g., E. coli)

- viruses (e.g., Rotavirus)

- protozoa (e.g., Giardia)

7.2 Quartz Glass Window

The UV module uses a high-transmission quartz glass window because:

- standard glass blocks UV-C

- quartz transmits > 90% at 254 nm

- it is heat-resistant

- chemically inert

The window is fully enclosed within a shielded housing.

7.3 UV Dose Calculation

For ES‑AQUA:

• 	target dose: ≥ 40 mJ/cm²

• 	minimum exposure time: 40–60 seconds

• 	dependent on:

• 	LED power

• 	distance to water

• 	flow rate

7.4 Minimum Exposure Time

The UV chamber is designed to ensure:

- slow enough flow

- complete exposure

- no shadow zones

Typical values:

- flow rate: 0.3–0.6 L/min

- contact time: ≥ 40 seconds

8. Theoretical Effectiveness Analysis

8.1 Mechanical Filtration

Removes:

• 	sand

• 	silt

• 	suspended solids

• 	turbidity

Effectiveness: high, depending on sand quality.

8.2 Activated Carbon Adsorption

Removes:

• 	chemicals

• 	pesticides

• 	odors

• 	taste compounds

• 	organic contaminants

Effectiveness: medium to high, depending on carbon quality

8.3 Microfiltration

Removes:

• 	bacteria (partially)

• 	microplastics

• 	fine particles

Effectiveness: medium, with optional membrane very high.

8.4 UV‑C Disinfection

Inactivates:

• 	bacteria: > 99.99%

• 	viruses: > 99.9%

• 	protozoa: > 99%

UV‑C is one of the most reliable water disinfection methods.

9. Graphic Design Guide

(This section describes the graphics so designers can reproduce them accurately.)

9.1 Exterior View

• 	cylindrical housing

• 	inlet at top, outlet at bottom

• 	blue‑white color scheme

• 	Auron/ES emblem on the front

9.2 Cross-Section View

From top to bottom:

- coarse gravel

- fine gravel

- quartz sand

- activated carbon

- microfiber layer

- optional membrane

- UV chamber

- Outlet

9.3 Functional Diagram

• 	arrows showing water flow

• 	UV chamber highlighted

• 	barrier chain color‑coded

9.4 Detail Graphics

• 	UV module

• 	quartz window

• 	activated carbon layer

• 	backwash valve

10. Deployment Areas & Humanitarian Impact

10.1 Villages

• 	household supply

• 	community wells

• 	schools

10.2 Camps

• 	refugee camps

• 	emergency shelters

• 	mobile water stations

10.3 Crisis Zones

• 	natural disasters

• 	epidemics

• 	conflict areas

10.4 Off‑Grid

• 	expeditions

• 	rural regions

• 	temporary settlements

11. Safety Sheet

11.1 Material Requirements

• 	food‑grade plastics only

• 	no toxic adhesives

• 	quartz sand instead of construction sand

11.2 UV Safety

- UV light must not escape

- quartz window fully shielded

- no direct line of sight to LED

11.3 Hygiene

• 	regular cleaning

• 	avoid touching filter layers with dirty hands

• 	do not inhale activated carbon dust

12. Status & Validation

12.1 Test-Ready

The system is ready for:

- laboratory testing

- NGO field trials

- governmental validation

12.2 Laboratory Requirements

- microbiological testing

- UV dose measurement

- flow rate measurement

- material analysis

12.3 Further Development

Planned improvements:

• 	optimized UV chamber

• 	modular filter cartridges

• 	solar‑powered option

• 	quick‑assembly system